Testing of “Text-to-Speech” System Developed in MSU

Alexey Babkin, Leonid Zakharov

Lomonosov Moscow State University, Vorobjovy Gory, MGU, 1-St Building Of The Humanities Moscow, 
Russia, 119 899

Abstract. The paper addresses the problem of evaluation of the quality of text-to-speech synthesis. Testing procedures for evaluating the main parameters of synthesized speech, i. e. intelligibility and naturalness are described together with the programs used for their computer implementation. The analysis of the results of the tests developed and carried out by the Speech group of the Philological Department of MSU is given. The results allow to make conclusion about the quality of the synthesis of the given system at the time of testing, and make a proper modification of the allophone database, and finally, to choose more suitable method to be used in speech generation module.

# 1. THE PROBLEM OF QUALITY EVALUATION IN systems of speech synthesis

The necessary stage in developing a system of speech synthesis is the evaluation (testing) of its quality. It helps to solve three main problems.

1. It provides information about the quality of a system as a whole. The results serve as a basis for taking a decision about the possible ways of its application. In this case the system is compared on the one hand to other speech synthesis systems and the conclusions are drawn about its competitiveness and on the other hand it is compared to natural speech (using identical material) and the conclusions are drawn about the possibility to use this system instead of natural speech.

2. It provides the system designers with the information about the system’s drawbacks and flaws at a given stage of development that should be eliminated at some later stage.

3. Sometimes the designers have to choose the best among the several varieties of the system’s working capacities. This may concern the system as a whole or only one of its main modules. The third problem is close to the first in its goals and to the second in its methodology. Several varieties of the system output must be compared and then the best one according to the results of the tests may be chosen.

For systems having to do with speech special testing procedures are needed to evaluate the quality of its work. These are speech tests, i. e. tests, built from the elements of human speech: sounds, syllables, words, phrases and texts. The tests help to find mistakes in a given system and at the same time they show the directions of its further development and improvement. One can test either the whole system or just some of its components.

As the problem of testing systems of speech synthesis has appeared rather recently, in this area the tests developed for other speech problems are usually used in particular test for estimating the quality of speech transmitted over communication channels. There are two basic directions in testing such systems: the tests for “intelligibility” and “naturalness” of speech, which are analyzed either together, or separately. The State Standard that came into force in 1997 “GOST R 50840-95 Speech transmission over various communication channels. Techniques for measurements of speech quality, intelligibility and voice identification” gives the definitions to these concepts.

Among the most popular methods of “intelligibility” evaluation are the following: word tests — Diagnostic rhyme test (DRT) and Modified rhyme test (MRT), various syllabic tests of a type Consonant-Vowel-Consonant (CVC) or Vowel-Consonant-Vowel (VCV). At the sentence level different phrasal tests are used, e. g. Semantically Unpredictable Sentences test (SUS). For testing “naturalness” various comparisons of pairs are used as well as Mean Opinion Score test (MOS).

The Philological Faculty of MSU has a long-standing experience in testing speech systems. The researchers of the Phonetic laboratory participated in the creation of GOSTs (the State Standards), including the one mentioned above, and produced their own tests for various tasks. Among them the quality evaluation experiments conducted by I. G. Frolova deserve special attention.

In some of them a method of multidimensional scaling was used, namely that of a semantically opposite pairs. In these experiments 7-point system was used to place the quality of the synthesized speech along the evaluative scales such as “nasal — pure”, “rude — tender”, “flowing — sharp” etc. (1).

In other experiments the quality of the synthesized speech was evaluated by determining the volume of the short-term auditory memory of the hearers and the dynamics of its change within a fairly long period of work. In one of the experiments it turned out, that the volume of the short-term auditory memory is diminished to a greater extent for the type of synthesis that shows better results in other tests. It reveals that auditors get tired sooner while working with this type of synthesis and thus points to certain limitations on its application (2).

Results of the investigations, obtained by MSU experimental phoneticians are analyzed in (3), where the corresponding generalizations are stated on their basis.

One of the authors of this paper, who took part in these investigations proposed a universal speech test for estimating the quality of the synthesized speech.

2. A UNIVERSAL TEST FOR ESTIMATING THE QUALITY OF TTS SYSTEMS

The resolution of the problem of optimal testing is confronted with two contradictory requirements: on the one hand the speech data must be as representative as possible, and as a consequence the volume of tests should be significantly increased, on the other hand the procedure of testing itself should be as short as possible, and as a consequence the volume of tests should be reduced. The problem was solved in the following way. For the maximum representativity of the speech samples the general corpus of tables is created. Each table serves for testing one sound in different positions within a word. For diminishing the testing time we created a diagnostic table that included only those words from the general corpus (two for each sound), in which we observe the largest number of the mistaken recognition of these sounds. The testing procedure goes like this. First the diagnostic test is used. If the sounds we are examining are recognized well within the words of a diagnostic table, one can assume that in other positions these sounds should be recognized as well. In case of mistaken recognition one has to address the table for a given sound from the general corpus in order to study in greater detail the intelligibility of this sound in different positions within a word.

As a result we have constructed a general corpus of tables, containing 2400 words (48 tables with 50 words in each). After that the material was recorded on a tape recorder, voiced by a speaker with standard pronunciation (orthoepic norms were under control) and later it was presented to each of the three groups of auditors (5 persons in each) in three regimes: without noise, with “white” noise (the proportion signal / sound S:N = 1:1), with “white” noise with the fall of 3 dB per octave (S:N = 1:1). Using the data obtained in these experiments we selected 2 words from each table, within which the sound under control was misrecognized most often. In this way we got a diagnostic table, including 96 (2  48) words.

3. THE DESCRIPTION OF THE EXPERIMENTS FOR THE EVALUATION 
OF THE QUALITY OF SPEECH SYNTHESIS

The choice of testing methods was determined mainly by the problems. that had to be solved with their help. The research was not aimed at the complex testing of the whole system of speech synthesis in all possible directions and by all possible parameters. We confined ourselves to methods, that allow to check the abilities of the signal generation module that were important for us and also to methods that allow to determine the directions of further development of speech synthesis.

Three tests were conducted — 2 for intelligibility and 1 for naturalness.

3.1. The 1st Intelligibility Test

In this test one word is presented to auditors and they have to identify it in a list of 5 similar words. We designed a table including 50 such lists (altogether 50  5= 250 words). In the allophonic database of the TTS system, developed at the Philological Faculty of MSU there are 55 different sounds (5), each of which is represented by several allophones depending on its position in a word and on its sound environment. The main principle of this test is that alternative words must differ only in one sound, i. e. we used “quasy-homonymes”. The additional advantage is the limited choice (auditors could choose an answer only among 5 given alternatives): only one particular sound was studied while all the accidental substitutions, caused by the “defects” in other sounds were excluded. We also diminished influence of frequency of words and word-forms on the hearers’ reactions (the hearer usually expects the words to be in their canonical vocabulary forms, e. g. for nouns — in nominative case, singular, while in our tests oblique forms of nouns predominate, often with prepositions, and the words may not be present in auditors’ active lexicon).

P    = [(48 9 2 – 42) : 48 9 2] 100% = 95,1%

Note that in this experiment we deal only with sound intelligibility (limited choice from a list of words)? or we can assume that word intelligibility equals sound intelligibility.

3.2. The 2nd Intelligibility Test

For this test a diagnostic table from the universal test described above was used (without any corrections). 96 words were synthesized and presented to the auditors in 2 regimes of signal generation (see 3.1). The crucial difference between this test and the previous one is the absence of limited choice. In such experiments any word may be a reaction to the stimulus, and the reaction may be simply accidental and may even have nothing in common with what was heard. This circumstance made the assessment of the results more difficult and deminished the diagnostic power of the test. On the other hand, it showed defects in those sounds (and in those positions of the sound within a word) that could be not appointed to be checked by the researches. Anyway even acoustically non-motivated sound mistakes in a word (given the honest work of auditors) usually serve as a signal that there is something wrong in the way the word sounds. In our experiment the situation was complicated by the fact that words in a test are given not in their “dictionary form”, being oblique case forms of nouns, finite verb forms etc. (cf. the advantages of the 1st test above) and that there is a number of rare words among them. In order to prevent the auditors from trying to “squeeze” what they hear into the dictionary form of a word that came to their mind, they were told in the instructions that the testing material contains “pseudo-words”, i. e. words, build in a way, appropriate to the structure of a given language, but not existing in that language, like words in Shcherba’s famouse “glokaja kuzdra…” example.

Psound = (4761 – 185) : 4761 = 96,1%

and

Pword    = (864 – 143) : 864 = 83,4%

where 4761 — the overall number of sounds in a test; 185 — a number of sounds that were misheard; 864 — a number of words in a test (96 words in a test, 9 auditors); 143 — a number of words that were misheard.

A fairly low word intelligibility is explained by the special complexity of the test we used (remember that the test was compiled on the basis of mistakes, made by auditors working with natural speech). As an optional result of this kind of testing one can consider the confirmation of the test’s diagnostic power: number of mistakes in sounds, that were checked, was 3 times as big as that in other sounds.

Sound intelligibility (independently of Signal Generation module regime) is sufficiently high and it shows good quality of the system with respect to intelligibility parameter. On the other hand we obtained valuable data for the further improvement of the TTS allophonic database. On the grounds of any sound substitution, made by auditors, one can replace or correct the corresponding allophone in the database (cf. substitutions like tsari+te  sori+te).

3. 3. Test for Naturalness

The aim of this test is to choose the best algorithm among those proposed as a base for modification of prosodic features in the Signal Generation module and to evaluate the level of naturalness of the synthesized speech. The main algorithms, that are implemented in Signal Generation module, are the following (6):

a) the simplest algorithm (based on “cut and slice” technology) – [0];

b) the simplest algorithm with the “smoothing out” of the end point – [1];

c) a complicated algorithm with a square window – [2];

d) a complicated algorithm with a triangle window – [3];

e) a complicated algorithm with a Hanning window – [4].

For evaluating naturalness of speech synthesis we decided to include in a test speech samples of real speaker, who served as a source of data for the acoustic database of our TTS system. These natural samples were given for comparison together with the artificial ones:

f) speech samples of real speaker – [5].

A test, that was designed, includes 250 words. According to the list of words synthesized samples were prepared using the above mentioned algorithms modifying the prosodic features of a vocal allophone. From the initial speech material, read by a speaker, the needed words were “cut out”. Both kinds of samples were grouped together according to the testing program requirements.

For evaluating naturalness we have chosen a method, based on a well-known method of pair comparisons and MOS-test. We used 4-point scale instead of a 5-point scale (having excluded the point “satisfactory”), because both the authors of the TTS system (researchers from MSU) and auditors (students of the Philological Faculty of MSU) are accustomed to it:

4 (Excellent)

3 (Good)

2 (Poor)

1 (Bad)

In this test we decided to unite the evaluation of naturalness with the choice of the best variant. Naturalness value is ascribed only to the best variant of pronunciation, thereby the best variant is determined uniquely.

In this test auditors have 2 words for evaluation. They must choose the best one of them and evaluate its naturalness using the 4-point scale.

The results lead to the following conclusions:

1) The most similar samples are [0] and [1], as we have expected;

2) natural samples differ from synthesized ones significantly, although from 3% to 6% of pairs do not have any perceptible difference: it can be explained by the fact, that the natural samples form the basis of the TTS system and that the system tested has good quality;

3) samples synthesized by different algorithms are very much alike, their difference does nor exceed 20%.

Interestingly, there is a group of words, for which auditors claimed that the synthesized sample is better than the natural one. The probability of such a valuation fluctuates from 7% to 14% for different algorithms. Average probability of such a valuation equals:

Pnat < synth = 100% – 5,2%    – 84,9% = 9,9%

This is a considerably high result. In other words, the quality of each 10th of the synthesized samples is considered higher that of the natural one.

REFERENCE

1. Frolova, I. G., Estestvennost' i razborchivost' kak parametry ocenki kachestva sintezirovannoj rechi, Eksperimental'naja fonetika, Moscow, MGU, 1989, pp. 95-102.

2. Frolova, I. G., O vozmozhnosti operativnogo testirovanija kachestva sinteticheskoj i cifrovoj rechi, Tezisy dokladov 16 Vsesojuznogo seminara “ARSO-16”, Moscow, 1991.

3. Rechevye testy i ix primenenie, Ed. L. V. Zlatoustova, Moscow, MGU. 1986.

4. Koval'chuk, I. I., Zinovieva, N. V., Zakharov, L. M., Opredelenie principov optimal'nogo testirovanija sistemy avtomaticheskogo raspoznavanija i sinteza rechi, Tezisy    dokladov 15 Vsesojuznogo seminara “ARSO-15”, Tallinn, 285-286, 1989.

5. Zakharov, L. M., Transkripcija tekstov pri sinteze i analize russkoj rechi (Text Transcription for Speech Synthesis and Analisis in Russian), Proceedings of International Workshop “Dialogue96” (Computational Linguistics and its Applications). Moscow, 88-91, 1996.

6. Babkin, A. V., Avtomaticheskij sintez rechi — problemy i    metody generacii rechevogo signala (Automatic synthesis of speech — problems and methods of speech signal generation), Proceedings of International Workshop “Dialogue98” (Computational Linguistics and its Applications), Kazan', 425-437, 1998.